Summary of Methods:
 Modeling Approach:

A multiperiod model was developed using a flowsheet that represents distributed-scale water treatment system including PV, wind, a battery, and the treatment unit. An energy balance was applied at each hourly time step to determine water production and energy use. The required water production over the 5-day window was fixed at 500 m³, equivalent to an average of 100 m³ per day. The treatment system was allowed to operate intermittently, turning on or off in response to available energy from generation or battery storage. The sizes of all components were treated as decision variables, making this a combined design and operational optimization problem.

The objective was to minimize total capital and operating expenditures across all components while meeting the water production target. WaterTAP flowsheets were used to determine the specific energy consumption (SEC), capital costs, and operating costs for several components. The model was formulated as a mixed-integer linear program (MILP) and solved using Gurobi.

Scenarios

Three treatment technologies were evaluated—RO, UF + IX, and UF + GAC—under two seasonal conditions, October and June. WaterTAP unit models for GAC and IX were used to estimate energy consumption and costs. UF energy consumption was estimated using an assumed pump efficiency of 80% and a required pressure differential based on water quality. SEC for RO was based on an existing RO train designed for similar feedwater. WaterTAP was not used to estimate RO system costs.

Poorer water quality in October was assumed to increase both SEC and operating costs for the treatment systems. Product water quality was not evaluated.

A comparison was made to a traditional off-grid treatment system sized exactly at 100 m³/day, operated continuously, and powered by a diesel generator.

Diesel generator costs were estimated using literature-based levelized cost of energy (LCOE) values.

A. Zein et al., 2023, Analysis of variable reverse osmosis operation powered by solar energy
E. Mulenga et al., 2023, Techno-economic analysis of off-grid PV-Diesel power generation system for rural electrification: A case study of Chilubi district in Zambia

An LCOE value of $0.35/kWh was used as the baseline case, which corresponds to diesel fuel costs of approximately $4 per gallon. An additional scenario assumed a 60% increase in diesel prices to reflect recent cost trends. Because the system operated continuously, no optimization was required; annual cost was calculated using:
Cost=SEC*LCOE*100 m3day*365 dayyr

Input Data:

Cost estimates for PV, wind, and battery systems were sourced from SAM, which derives values from the Annual Technology Baseline report.
Cost data for treatment technologies were obtained from WaterTAP unit models or existing studies.
PV and wind power outputs were based on a Texas load profile generated through SAM.
The feedwater was surface water, and no specific TDS value was assigned, as it did not affect UF, GAC, or IX performance in the model.
(Additional details are available in the third tab of the results spreadsheet.)